Project Report: Autonomous RFID Patrol & Inventory Robot

Course: AI & DS 2001 - Programming for AI & Intro to DS

Date: December 07, 2025

Group Members:

- Abdul Ghafoor – 24I-0118

- Haseeb Ullah – 24I-0086

- Muhammad Faisal – 24I-6077

- Waiz Afzal – 24I-6536

- Talha Habib – 24I-0029

1. Introduction

This project falls under the domain of Physical AI and Internet of Things (IoT). We have designed an autonomous mobile robot capable of navigating a specific route, stopping at designated checkpoints, scanning RFID tags (simulating inventory or security checkpoints), and uploading the data in real-time to a cloud database (Google Sheets).

The system integrates hardware components (Motors, RFID sensors) with software logic (C++ algorithms, HTTP protocols) to create a functional automated agent.

2. Key Features

As required, the robot implements complex features involving navigation and data processing:

- Autonomous Route Navigation: The robot follows a pre-programmed timed path including forward movement, stops, and a complex U-Turn maneuver (Bridge crossing simulation).

- Wireless RFID Scanning: utilizing the MFRC522 sensor to detect Unique Identifiers (UIDs) from cards/tags during stops.

- IoT Cloud Integration: Real-time data logging via WiFi. The robot acts as an HTTPS client to POST scanned data to a Google Script macro.

- Audio Status Indicators: The system generates distinct beep sounds to provide feedback for startup, successful RFID scanning, and mission completion, allowing for "headless" operation without a display.

3. System Architecture & Modules

3.1 Hardware Modules

- Microcontroller: ESP32 (Selected for WiFi capabilities).

- Motor Driver: L298N H-Bridge (Controls 4 DC motors).

- Sensor: MFRC522 RFID Reader (SPI Communication).

- Actuators: 4x DC Motors (4WD Chassis).

- Power: Li-ion Battery Pack.

3.2 Block Diagram

The system is centered around the ESP32 microcontroller, which manages all peripherals. The L298N driver handles high-current power distribution to the motors based on logic signals from the ESP32. The MFRC522 communicates via the SPI protocol for high-speed tag reading. Finally, the ESP32 utilizes its built-in WiFi module to transmit processed data to the Google Cloud ecosystem via HTTPS requests.

3.3 Circuit Diagram & Connections

The hardware is assembled with the following pin mapping based on the code:

- Motors (L298N):

  - Enable Pins: ENa (GPIO 14), ENb (GPIO 32).

  - Left Motor Logic: IN1 (27), IN2 (26).

  - Right Motor Logic: IN3 (25), IN4 (23).

- RFID (SPI):

  - SDA (SS): GPIO 5

  - SCK: GPIO 18, MOSI: GPIO 19, MISO: GPIO 22

  - RST: GPIO 23

- Audio Output: GPIO 4 (Controls beep signal)

4. Functionality & Algorithm

The software processes sensor data and controls hardware output through a specific state machine loop.

4.1 Navigation Logic (The "Legs")

The robot does not move randomly; it follows a strict algorithmic path:

- Leg 1: Executes a loop 3 times. Move Forward (6s)  Stop & Scan (4s).

- U-Turn Maneuver: Performs a complex "Bridge" turn: Rotate Right  Move Across (3s)  Rotate Right (to face return path).

- Leg 2: Executes the return loop 3 times. Move Forward (6s)  Stop & Scan (4s).

4.2 RFID & Data Processing

When the robot enters the stopAndScan() state:

- The motors stop, but the processor remains active.

- The MFRC522 checks for new cards.

- Algorithm: If a card is detected, the UID bytes are converted to a Hex String.

- Output: The system generates a beep sound confirmation, and the sendData() function is triggered to upload the tag ID.

4.3 Cloud Communication

The sendData(String uid) function utilizes the HTTPClient library. It constructs a JSON payload {"uid": "..."} and sends a POST request to a deployed Google Apps Script URL. This ensures data persistency beyond the robot's physical memory.

5. Visual Evidence

5.1 Simulation

5.2 Physical Implementation (BTS)

5.3 Working Output

6. Work Division

- Haseeb Ullah: Hardware assembly, soldering, and chassis construction.

- Muhammad Faisal: Circuit design and power management (L298N integration).

- Waiz Afzal: Software implementation (Motion control algorithms).

- Talha Habib: Software implementation (RFID integration).

- Abdul Ghafoor: Report generation and Software implementation (integrating Google Sheets Database).

7. Conclusion

We successfully designed and implemented a physical AI robot that meets the semester project requirements. The robot autonomously navigates, gathers environmental data (RFID tags), and processes this data via the cloud, demonstrating a complete IoT pipeline.

Project Code

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <SPI.h>

#include <MFRC522.h>

// ==========================================

// 1. SETTINGS

// ==========================================

const char* ssid = ""; // WiFi Credentials removed for security

const char* password = "";

String GOOGLE_SCRIPT_URL = "https://script.google.com/macros/s/AKfycbwgg-URroK90rq6HM8e79DnuYu5sgyjIWwHeiWiSZQlRNR3zd2gTbn1kJqSnBaZC8GsBw/exec";

// Timing Settings (Milliseconds)

const int MOVE_TIME = 3000;    // 6 Seconds Move

const int STOP_TIME = 4000;    // 4 Seconds Stop/Scan

const int GAP_MOVE_TIME = 3000;// 3 Seconds (The middle bridge)

const int TURN_TIME = 400;     // Time to rotate 90 degrees

// ==========================================

// 2. PINS

// ==========================================

// Motors

const int ENa=14;

const int ENb=32;

const int IN1 = 27; const int IN2 = 26; // Left

const int IN3 = 25; const int IN4 = 23; // Right

// RFID

const int RFID_SS_PIN = 5;

const int RFID_SCK_PIN = 18;

const int RFID_MOSI_PIN = 19;

const int RFID_MISO_PIN = 22;

const int RFID_RST_PIN = 23;

// Buzzer

const int BUZZER_PIN = 4;

MFRC522 rfid(RFID_SS_PIN, RFID_RST_PIN);

bool routeComplete = false; // Flag to stop after finishing

void setup() {

Serial.begin(115200);

analogWrite(ENa, 200);

analogWrite(ENb, 255);

// Init Hardware

pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

SPI.begin(RFID_SCK_PIN, RFID_MISO_PIN, RFID_MOSI_PIN, RFID_SS_PIN);

rfid.PCD_Init();

// Connect WiFi

WiFi.begin(ssid, password);

Serial.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500); Serial.println(".");

}

Serial.println("\nWiFi Connected!");

// Signal Start

beep(3);

delay(2000); // 2 second delay before the mission starts

}

void loop() {

if (routeComplete) {

return;

}

// --- LEG 1: The First 3 Stops ---

Serial.println("--- STARTING LEG 1 ---");

for (int i = 1; i <= 3; i++) {

Serial.print("Segment "); Serial.println(i);

moveForward();

delay(4000);

stopAndScan(STOP_TIME);

}

// --- THE U-TURN MANEUVER ---

Serial.println("--- PERFORMING TURN ---");

rotateRight();

delay(TURN_TIME);

moveForward();

delay(GAP_MOVE_TIME);

rotateRight();

delay(TURN_TIME);

// --- LEG 2: The Return 3 Stops ---

Serial.println("--- STARTING LEG 2 ---");

for (int i = 1; i <= 3; i++) {

Serial.print("Segment "); Serial.println(i);

moveForward();

delay(4000);

stopAndScan(STOP_TIME);

}

// --- MISSION COMPLETE ---

Serial.println("--- ROUTE FINISHED ---");

stopCar();

beep(5);

routeComplete = true;

}

// ==========================================

// CUSTOM FUNCTIONS

// ==========================================

void stopAndScan(int duration) {

stopCar();

beep(1);

unsigned long startTime = millis();

while (millis() - startTime < duration) {

if (rfid.PICC_IsNewCardPresent() && rfid.PICC_ReadCardSerial()) {

String uidString = "";

for (byte i = 0; i < rfid.uid.size; i++) {

uidString += String(rfid.uid.uidByte[i] < 0x10 ? " 0" : " ");

uidString += String(rfid.uid.uidByte[i], HEX);

}

uidString.toUpperCase();

Serial.println("Scanned: " + uidString);

digitalWrite(BUZZER_PIN, HIGH); delay(100); digitalWrite(BUZZER_PIN, LOW);

sendData(uidString);

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

}

}

}

void sendData(String uid) {

if (WiFi.status() == WL_CONNECTED) {

WiFiClientSecure client;

client.setInsecure();

HTTPClient http;

http.begin(client, GOOGLE_SCRIPT_URL);

http.setFollowRedirects(HTTPC_STRICT_FOLLOW_REDIRECTS);

http.addHeader("Content-Type", "application/json");

String payload = "{\"uid\": \"" + uid + "\"}";

int httpCode = http.POST(payload);

if (httpCode > 0) {

Serial.println("Data Uploaded.");

}

http.end();

}

}

void moveForward() {

digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

}

void rotateRight() {

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

}

void stopCar() {

digitalWrite(IN1, LOW); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW); digitalWrite(IN4, LOW);

}

void beep(int times) {

for(int i=0; i<times; i++){

digitalWrite(BUZZER_PIN, HIGH); delay(100); digitalWrite(BUZZER_PIN, LOW); delay(100);

}

}